1. Identification of Product

Product Identifier Product Name: 4-Methylthio-3-trifluoromethylphenylisocyanate Product Code: IC00152 CAS Number: 61386-59-2 MFCD Number: MFCD31009206 Relevant Identified Uses of Substance or Mixture and Uses Advised Against Research and Development use Supplier Details of the Safety Data Sheet Atlantic Research Chemicals Ltd, Unit A, Stratton View Business Park, Holsworthy Road, Bude, Cornwall EX23 9NR Telephone: +44 (0)1288 355370 E-mail: info@atlantic-chemicals.com

2. Hazards Identification

Classification of the Substance or Mixture Acute toxicity, oral (category 3) Acute toxicity, dermal (category 3) Acute toxicity, inhalation (category 2) Respiratory sensitisation (category 1) Skin sensitisation (category 1) Skin irritation (category 2) Eye irritation (category 2) STOT single exposure (category 3) Label Elements Signal Word: Warning Hazard Statements H301 - Toxic if swallowed. H311 - Toxic in contact with skin. H330 - Fatal if inhaled. H315 - Causes skin irritation. H317 - May cause an allergic skin reaction. H319 - Causes serious eye irritation. H334 - May cause allergy or asthma symptoms or breathing difficulties if inhaled. H335 - May cause respiratory irritation. Precautionary Statements P260 – Do not breathe dust/fumes/gas/mist/vapours/spray. P280 – Wear protective gloves/protective clothing/eye protection/face protection. P301 + P310 - IF SWALLOWED: Immediately call a POISON CENTER or doctor/ physician. P302 + P352 – IF ON SKIN: Wash with plenty of soap and water. P304 + P340 – IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. P305 + P351 + P338 – IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do, continue rinsing. Other Hazards Lachrymator (substance which increases the flow of tears).

3. Composition/Information on Ingredients

Substances 4-Methylthio-3-trifluoromethylphenylisocyanate Assay: 95% CAS number: 61386-59-2 Concentration Product Name CAS number: EC/EINECS:

4. First Aid Measures

Description of First Aid Measures Skin Contact IF ON SKIN: Wash with plenty of soap and water. If skin irritation occurs: Get medical advice/attention. Eye Contact If eye irritation persists: Get medical advice/attention. IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do so. Continue rinsing. Ingestion IF SWALLOWED: Immediately call a POISON CENTRE or doctor/physician. Do not induce vomiting. Inhalation IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. Most Important Symptoms and Effects No data. Treat symptomatically. Indication of any Immediate Medical Attention No additional measures required.

5. Fire Fighting Measures

Extinguishing Media Suitable Carbon dioxide. Dry chemical powder or foam. Sand. Unsuitable Water. Special Hazards Arising from the Substance or mixture In combustion emits carbon oxides, nitrogen oxides and hydrogen chloride. Advice for Fire Fighters Wear self-contained breathing apparatus. Wear protective clothing to prevent contact with skin and eyes.

6. Accidental Released Measures

Personal Precautions Do not attempt to take action without suitable protective clothing – see section 8 of SDS. Do not create dust. Evacuate the area immediately. Environmental Precautions Do not discharge into drains or rivers. Contain spillage with bunds. Methods & Materials Mix with sand or vermiculite. Transfer to a closable, labelled salvage container for disposal by an appropriate method.

7. Handling and Storage

Personal Precautions Safe Handling Ensure there is sufficient ventilation of the area. Protection against Do not eat, drink or smoke when using this product. explosions and fires Remove/take off immediately all contaminated clothing. Take off contaminated clothing and wash before reuse. Wash hands thoroughly after handling. Conditions for Safe Storage, Including any Incompatibilities Managing storage risks Store in a cool, well-ventilated area. Keep container tightly closed. Storage controls Do not store in conditions of high (or highly fluctuating) temperature. Store away from water. Store away from oxidising agents. Maintaining integrity Keep container tightly closed. Moisture sensitive - keep dry. Store under an inert atmosphere. Other advice Store in a well-ventilated place. Specific End Uses The end use(s) have not been fully determined. The substance is supplied for Research and Development purposes by professionals only.

8. Exposure Controls/Personal Protection

Control Parameters This product, as supplied, does not contain any hazardous materials with occupational exposure limits established by the region specific regulatory bodies. Exposure Controls General protective and Wear protective gloves/protective clothing/eye protection/face protection. hygiene measures Wash hands during breaks and at the end of handling the material. Immediately remove any contaminated clothing. Engineering measures Use only outdoors or in a well-ventilated area. Eye/face protection Safety glasses with side shields. Hand protection Protective gloves. Respiratory protection Do not breathe dust/fumes/gas/vapours/spray. Use breathing protection with high concentration. Skin protection Handle with gloves. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practice. Wash and dry hands. Protective clothing. Other personal protection No data. advice

9. Physical and Chemical Properties

Physical and Chemical Properties Molecular formula: C9 H6 F3 N O S Molecular weight: 233.212 Information on basic physical and chemical properties a) Appearance: Off-white solid b) Odour: No data c) Odour threshold: No data d) pH: No data e) Melting point/freezing point: 27-29 °C f) Initial boiling point and boiling range: No data g) Flash point: No data h) Evaporation rate: No data i) Flammability (solid, gas): No data j) Upper/lower flammability or explosive limits: No data k) Vapour pressure: No data l) Vapour density: No data m) Relative density: No data n) Water solubility: Hydrolyses o) Partition coefficient: n-octanol/ water: No data p) Auto-ignition temperature: No data q) Decomposition temperature: No data r) Viscosity: No data s) Explosive properties: Product is not explosive t) Oxidizing properties: No data Other Information No additional information.

10. Stability and Reactivity

Reactivity No unusual reactivity. Stability Stable under normal conditions. Possibility of Hazardous Reactions No hazardous reactions known. Conditions to avoid Do not expose material to any source of ignition. Excessive heat. Avoid moist atmosphere. Incompatible Materials Strong oxidizing agents. Water. Bases. Alcohols. Amines. Hazardous Decomposition Products In combustion emits carbon oxides, nitrogen oxides and hydrogen chloride.

11. Toxicology Information

Information Acute toxicity: Fatal if inhaled. Skin corrosion/irritation: Causes skin irritation. Serious eye damage: Causes serious eye irritation. Respiratory or skin sensitisation: May cause sensitisation by inhalation and skin contact. Germ cell mutagenicity: Not known. Carcinogenicity: There are no known carcinogenic chemicals in this product. Reproductive toxicity: Not known. Aspiration hazard: Not known. Additional To the best of our knowledge the acute and chronic toxicity of this substance is not fully known. No classification data on carcinogenic properties of this material is available from the EPA, IARC, NTP, OSHA or ACGIH.

12. Ecological Information

Toxicity Not known. Persistence and Degradability Not known. Bio-Accumulative Potential Not known. Mobility in Soil Not known. Results of PBT & PvB Assessment Not known. Other Adverse Effects Water hazard class 1 (self-assessment): slightly hazardous for water. Do not allow product to reach ground water, water bodies or sewage system, even in small quantities.

13. Exposure Controls/Personal Protection

Waste Treatment Methods Disposal operations Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber. Disposal of packaging Disposal must be made according to official regulations.

14. Transport Information

Air (ICAO) - UN Number: UN2250 - UN Proper shipping name: Diclorophenyl Isocyantes - Transport hazard class(es): 6.1 - Packing group: II - Environmental hazards: No - Special precautions for user: Warning: toxic substances. 7. Transport in bulk: Not applicable. Road (ADR) - UN Number: UN2250 - UN Proper shipping name: Diclorophenyl Isocyantes - Transport hazard class(es): 6.1 (T2) toxic substances. - Packing group: II - Environmental hazards: No - Special precautions for user: Warning: toxic substances. 7. Transport in bulk: Excepted quantities (EQ): E4 Limited quantities (LQ): 500 g Transport category: 2 Tunnel restriction code: D/E Sea (IMDG) - UN Number: UN2250 - UN Proper shipping name: Diclorophenyl Isocyantes - Transport hazard class(es): 6.1 - Packing group: II - Environmental hazards: Marine pollutant: no. - Special precautions for user: Warning: Toxic substances. 7. Transport in bulk: Not applicable.

15. Safety, Health and Environmental Regulations

Safety, Health and Environmental Regulations: Product is not subject to any additional regulations. Safety Assessment No chemical Safety Assessment

16. Other Information

ADR: Accord Europeen sur le transport des merchandises Dangereuses par Route (European Agreement concerning the International Carriage of Dangerous Goods by Road) RID: Reglement International concernant le transport des merchandises dangereuses par chemin de fer (Regulations concerning the International transport of Dangerous Goods by Rail) IMDG: International Maritime Code for Dangerous Goods IATA: International Air Transport Association IATA-DGR: Dangerous Goods Regulations by the International Air Transport Association ICAO: International Civil Aviation Organisation ICAO-TI: Technical Instructions by the ICAO GHS: Globally Harmonized System of Classification and Labelling of Chemicals CAS: Chemical Abstracts Service Associated Risk Phrases According to European Directives 67/548/EEC R23/24/25; Toxic by inhalation, in contact with skin and if swallowed. R36/37/38; Irritant to eyes, respiratory system and skin. R42/43; May cause sensitisation by inhalation and skin contact. Disclaimer The product is listed for research and development purposes only and not for human or animal use. As such, in most cases, the toxicological, ecological and physicochemical properties have not been fully determined and the product should be treated with respect and always handled under suitable conditions by appropriately qualified personnel. The responsible party shall use this datasheet only in conjunction with other sources of information gathered by them, and should make an independent judgement of suitability, to ensure proper use and protect the health and safety of employees. This information is furnished without warranty and any use of the product not in conformance with this safety data sheet, or in combination with any other product or process is the responsibility of the user. Certified by: J A Forshaw