1. Identification of Product

Product Identifier Product Name: 2-Aminonicotinaldehyde Product Code: AH00238 CAS Number: 7521-41-7 MFCD Number: MFCD01830382 EINECS: Unlisted Relevant Identified Uses of Substance or Mixture and Uses Advised Against Research and Development use Supplier Details of the Safety Data Sheet Atlantic Research Chemicals Ltd, Unit A, Stratton View Business Park, Holsworthy Road, Bude, Cornwall EX23 9NR Telephone: +44 (0)870 7746454 Fax: +44 (0)870 7746455 E-mail: info@atlantic-chemicals.com

2. Hazards Identification

Classification of the Substance or Mixture Acute toxicity, oral (category 4) Skin irritation (category 2) Eye irritation (category 2) STOT single exposure (category 3) Skin sensitisation (category 1) Label Elements Signal Word: Warning Hazard Statements H302 – Harmful if swallowed. H319 – Causes serious eye irritation. H335 – May cause respiratory irritation. H315 – Causes skin irritation. H317 – May cause an allergic skin reaction. Precautionary Statements P260 – Do not breathe dust/fumes/gas/mist/vapours/spray. P280 – Wear protective gloves/protective clothing/eye protection/face protection. P301 + P312 – IF SWALLOWED: Call a POISON CENTER or doctor/physician if you feel unwell. P302 + P352 – IF ON SKIN: Wash with plenty of soap and water. P304 + P340 – IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. P305+ P351 + P338 – IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do, continue rinsing. Other Hazards No information available.

3. Composition/Information on Ingredients

Substances 2-Aminonicotinaldehyde Assay: 97 % CAS number: 7521-41-7 Mixtures Concentration Product Name CAS number: EC/EINECS:

4. First Aid Measures

Description of First Aid Measures Skin Contact IF ON SKIN: Wash with plenty of soap and water. If skin irritation occurs: Get medical advice/attention. Eye Contact If eye irritation persists: Get medical advice/attention. IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do so. Continue rinsing. Ingestion IF SWALLOWED: Immediately call a POISON CENTRE or doctor/physician. Inhalation IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing. Most Important Symptoms and Effects No data. Indication of any Immediate Medical Attention No additional measures required.

5. Fire Fighting Measures

Extinguishing Media Suitable Carbon dioxide. Dry chemical powder or foam. Sand. Water spray. Unsuitable Water jet. Special Hazards Arising from the Substance or mixture In combustion emits carbon oxides and nitrogen oxides. Advice for Fire Fighters Wear self contained breathing apparatus. Wear protective clothing to prevent contact with skin and eyes.

6. Accidental Released Measures

Personal Precautions Do not attempt to take action without suitable protective clothing – see section 8 of SDS. Do not create dust. Evacuate the area immediately. Environmental Precautions Do not discharge into drains or rivers. Methods & Materials Mix with sand or vermiculite. Transfer to a closable, labelled salvage container for disposal by an appropriate method.

7. Handling and Storage

Personal Precautions Safe Handling Ensure there is sufficient ventilation of the area. Avoid breathing vapors, mist or gas. Protection against Do not eat, drink or smoke when using this product. explosions and fires Remove/take off immediately all contaminated clothing. Take off contaminated clothing and wash before reuse. Wash hands thoroughly after handling. Conditions for Safe Storage, Including any Incompatibilities Managing storage risks Store in a cool, well ventilated area. Keep container tightly closed. Storage controls Do not store in conditions of high (or highly fluctuating) temperature. Maintaining integrity Keep container tightly closed. Other advice Store in a well ventilated place. Specific End Uses The end use(s) have not been fully determined. The substance is supplied for Research and Development purposes by professionals only.

8. Exposure Controls/Personal Protection

Control Parameters Contains no substances with occupational exposure limit values. Exposure Controls General protective and Wear protective gloves/protective clothing/eye protection/face protection. hygiene measures Wash hands during breaks and at the end of handling the material. Immediately remove any contaminated clothing. Engineering measures Use only outdoors or in a well ventilated area. Eye/face protection Safety glasses with side shields. Hand protection Protective gloves. Respiratory protection Do not breathe dust/fumes/gas/vapours/spray. Use breathing protection with high concentration. Skin protection Handle with gloves. Dispose of contaminated gloves after use in accordance with applicable laws and good laboratory practice. Wash and dry hands. Protective clothing. Other personal protection No data. advice

9. Physical and Chemical Properties

Physical and Chemical Properties Appearance: Off white to pale yellow solid. Odour: No data. Melting point: 100-102 °C Boiling point: No data. Molecular formula: C6 H6 N2 O Molecular weight: 122.1260 Density: No data. Other Information No additional information.

10. Stability and Reactivity

Reactivity No unusual reactivity. Stability Stable under normal conditions. Possibility of Hazardous Reactions No hazardous reactions known. Conditions to avoid Do not expose material to any source of ignition. Avoid moist atmosphere. Incompatible Materials Strong oxidising agents. Hazardous Decomposition Products In combustion emits carbon oxides and nitrogen oxides.

11. Toxicology Information

Information Acute toxicity: No data. Skin corrosion/irritation: No data. Serious eye damage: No data. Respiratory or skin sensitisation: No data. Germ cell mutagenicity: Not known. Carcinogenicity: IARC: No component of this product present at levels greater than or equal to 0.1% is identified as probable, possible or confirmed human carcinogen by IARC. Reproductive toxicity: Not known. Aspiration hazard: Not known. Additional To the best of our knowledge the acute and chronic toxicity of this substance is not fully known. No classification data on carcinogenic properties of this material is available from the EPA, IARC, NTP, OSHA or ACGIH.

12. Ecological Information

Toxicity Not known. Persistance and Degradability Not known. Bio-Accumulative Potential Not known. Mobility in Soil Not known. Results of PBT & PvB Assessment Not known. Other Adverse Effects Do not allow product to reach ground water, water bodies or sewage system, even in small quantities.

13. Exposure Controls/Personal Protection

Waste Treatment Methods Disposal operations Offer surplus and non-recyclable solutions to a licensed disposal company. Dissolve or mix the material with a combustible solvent and burn in a chemical incinerator equipped with an afterburner and scrubber. Disposal of packaging Disposal must be made according to official regulations.

14. Transport Information

Air (ICAO) - UN Number: Not regulated. - UN Proper shipping name: Not applicable. - Transport hazard class(es): Not applicable. - Packing group: Not applicable. - Environmental hazards: No. - Special precautions for user: Not applicable. - Transport in bulk: Not applicable. Road (ADR) - UN Number: Not regulated. - UN Proper shipping name: Not applicable. - Transport hazard class(es): Not applicable. - Packing group: Not applicable. - Environmental hazards: No. - Special precautions for user: Not applicable. - Transport in bulk: Not applicable. Sea (IMDG) - UN Number: Not regulated. - UN Proper shipping name: Not applicable. - Transport hazard class(es): Not applicable. - Packing group: Not applicable. - Environmental hazards: Marine pollutant: no. - Special precautions for user: Not applicable. - Transport in bulk: Not applicable.

15. Safety, Health and Environmental Regulations

Safety, Heath and Environmental Regulations: Product is not subject to any additional regulations. Safety Assessment No chemical Safety Assessment

16. Other Information

ADR: Accord Europeen sur le transport des merchandises Dangereuses par Route (European Agreement concerning the International Carriage of Dangerous Goods by Road) RID: Reglement International concernant le transport des merchandises dangereuses par chemin de fer (Regulations concerning the International transport of Dangerous Goods by Rail) IMDG: International Maritime Code for Dangerous Goods IATA: International Air Transport Association IATA-DGR: Dangerous Goods Regulations by the International Air Transport Association ICAO: International Civil Aviation Organisation ICAO-TI: Technical Instructions by the ICAO GHS: Globally Harmonized System of Classification and Labelling of Chemicals CAS: Chemical Abstracts Service Associated Risk Phrases According to European Directives 67/548/EEC R22; Harmful if swallowed. R36/37/38; Irritating to eyes, respiratory system and skin. R43; May cause sensitisation by skin contact. Disclaimer The product is listed for research and development purposes only and not for human or animal use. As such, in most cases, the toxicological, ecological and physicochemical properties have not been fully determined and the product should be treated with respect and always handled under suitable conditions by appropriately qualified personnel. The responsible party shall use this datasheet only in conjunction with other sources of information gathered by them, and should make an independent judgement of suitability, to ensure proper use and protect the health and safety of employees. This information is furnished without warranty and any use of the product not in conformance with this safety data sheet, or in combination with any other product or process is the responsibility of the user. Certified by Tony Forshaw